| QUALITY TEAM · POWER TECHNICAL SUPPORT LV & MV Cable Jointing Inspection Checklist Inspection & Quality Checklist | TAQA DISTRIBUTION |
| --- | --- |

| REFERENCE NO. | REVISION | LAST UPDATED | DISCIPLINE |
| --- | --- | --- | --- |
| QT-PTS-CHK-001 | 2.0 | 08 Jul 2026 | Power — Cable Jointing |

| PROJECT / CONTRACT | LOCATION / FEEDER | CONTRACTOR | DATE OF INSPECTION |
| --- | --- | --- | --- |

1.  PRE-JOINTING PREPARATION

| # | Check item | Yes/No | N/A | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Correct joint kit selected for cable type, size and voltage class |  |  |  |
| 2 | All materials within shelf life and undamaged; packaging intact until use |  |  |  |
| 3 | Jointing enclosure / tent erected; area clean, dry and protected from weather |  |  |  |
| 4 | Cable ends undamaged; correct overlap and working length available |  |  |  |
| 5 | Correct tooling, crimp tool and dies available and calibrated where required |  |  |  |
| 6 | Approved drawing / manufacturer instruction available at the work face |  |  |  |
| 7 | PPE and safe isolation confirmed before work begins |  |  |  |

2.  JOINTING

| # | Check item | Yes/No | N/A | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Cable preparation dimensions set out per manufacturer instruction |  |  |  |
| 2 | Insulation and semi-con screen cut cleanly, no nicks to conductor |  |  |  |
| 3 | Conductor connector correct size; crimped with correct die and number of crimps |  |  |  |
| 4 | Connector alignment and inter-phase spacing correct |  |  |  |
| 5 | Insulation / stress control applied without voids, trapped air or contamination |  |  |  |
| 6 | Metallic screen continuity maintained and bonded as specified |  |  |  |
| 7 | Earth continuity conductor connected and continuous across the joint |  |  |  |

3.  POST-JOINTING & RECORDS

| # | Check item | Yes/No | N/A | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Joint protection / mechanical shell installed correctly |  |  |  |
| 2 | Joint position, phasing and identification recorded |  |  |  |
| 3 | Photographic record taken at key stages |  |  |  |
| 4 | Backfill / protection reinstated as specified |  |  |  |
| 5 | Work area cleared; waste and offcuts removed |  |  |  |

Mark each item Yes (compliant), No (non-conformance — raise on the remarks line) or N/A. Any “No” must be closed out before the activity is accepted.

SIGN-OFF

| INSPECTED BY (NAME / SIGNATURE) | REVIEWED BY — QUALITY TEAM | DATE |
| --- | --- | --- |