Cable Laying Checklist

Date:

| Contractor name |  |
| --- | --- |
| Consultant Name |  |
| Contract /PO No |  |
| Scheme No. / WO No. |  |
| Work location |  |
| Cable LV/11/22/33 KV |  |
| survey drawing to be attached. |  |

| SN | Content | YES | NO | NA | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | Availability of approved project documents such as cable route, method statement and risk assessment. |  |  |  |  |
| 2 | Cable Inspection: Physical damage or deformity checked. |  |  |  |  |
| 3 | Bending Radius: Within manufacturer’s guidelines |  |  |  |  |
| 4 | Availability of Cable Pulling winch machine and operator along with certificates. Pulling Tension: Controlled to avoid overstress. |  |  |  |  |
| 5 | Roller Usage: Rollers to be placed within 2 meters for smooth pulling. rollers to be placed on each trench edge/route curves and on ducts entries to avoid outer jacket scratch and damages |  |  |  |  |
| 6 | Sufficient cable length for rework jointing and termination. |  |  |  |  |
| 7 | Cable outer sheet for laid cable has no noticeable damages due to pulling. |  |  |  |  |
| 8 | LDPE tiles and warning tapes: Laid above cables (as per the approved trench cross section). |  |  |  |  |
| 9 | Sealing ends: cables shall be sealed at each end proper end caps. |  |  |  |  |
| 10 | Backfilling to be done as per TAQA specification and as per the approved cross-sectional layout. Compaction to be done for the final layer. |  |  |  |  |
| 11 | Cable / Joint marker: Installed at every 50 meters interval and at all direction change. |  |  |  |  |
| 12 | Gutch material to be provided and compacted throughout the cable route for the sandy area. |  |  |  |  |
| 13 | Required pictures as proof are collected? |  |  |  |  |
| 14 | Cable sheath test for 22kV, 33kV after laying cable to be verified of sheath integrity. Test kit calibration certificate to be verified. |  |  |  |  |

Note: the contractor must follow all steps mentioned in CONSTRUCTION MANUAL – SECTION II latest version.

| Contractor Representative |  | Supervision Team |  |
| --- | --- | --- | --- |
| Name |  | Name |  |
| Signature |  | Signature |  |